# APPROVAL NOTE — TK-4102

Ref: MRPL/INSP/2026/0412    MOC No: MOC-2026-0231

# Deviations

Shell plate thickness is 11.2 mm (below nominal 12.0 mm). Minimum required thickness per API 653 calculation is 10.4 mm, therefore the vessel remains fit for service until next turnaround.

PSV-2041 set at 12.5 barg against SOP-114 requirement of 14.0 barg. This is a non-conformance and must be corrected before restart.

# Decision

Conditional approval granted for continued operation until 31-Mar-2027, subject to: (1) quarterly UT monitoring at course-1, (2) PSV reset within 14 days, (3) re-inspection before any change in service.

Approved by: A. Deshpande, Head — Inspection    Date: 13-Sep-2026