MANGALORE REFINERY AND PETROCHEMICALS LIMITED

# NON-CONFORMANCE REPORT

| NCR Number | NCR-2026-044 |
| --- | --- |
| Date Raised | 09-Mar-2026 |
| Unit | Unit 24 - Naphtha Hydrotreater |
| Equipment | PSV-2422 |
| Reference Certificate | NHT-PSV-2026-121 dated 08-Mar-2026 |
| Status | OPEN |
| Deviation Approval | NONE. No deviation has been applied for or granted for PSV-2422. |

## Description of Non-Conformance

Relief valve PSV-2422 (nameplate set pressure 12.0 barg) recorded a governing as-found popping pressure of 12.60 barg during shop testing on 08-Mar-2026. The applicable tolerance is plus or minus 3 percent of set pressure per API 527.

## Disposition

PSV-2422 shall NOT be returned to service on the strength of the 08-Mar-2026 certificate. The valve is to be re-tested after cleaning and lapping, and an acceptable as-left result recorded before re-installation.

Note for clarity: deviation approval DEV-2026-017 covers E-2404 shell thickness only. It does not cover PSV-2422, and shall not be cited in support of returning this valve to service.

Raised by: D. Shenoy, Inspection Department

Approved by: K. R. Prabhu, Head - Technical Services